# Cyclone V Custom PHY with automatic synchronization state machine design example

## Overview

This basic design example with Modelsim simulation demonstrates the implementing of Cyclone V Custom PHY with automatic synchronization state machine. The purpose of this design example is to assist users to have quick start with the Cyclone V transceivers. The design consist of only one transceiver channel with data pattern switch between synchronization and incremental pattern. The design also come with example test bench and TCL file to run simulation in Modelsim for reference.

To run the simulation, do the following:

1. Unzip the files 
2. Change the Modelsim directory to the unzipped folder 
3. Type "source simulation_setup.tcl" 
4. Type "ld" to compile 
5. Type "simulate" to start simulation

## Design File

Cyclone V Custom PHY with automatic synchronization state machine design example QII v15.0 (ZIP)

## Design Specifications

The table below lists the specifications for this design:

| Attribute | Specification |
| --- | --- |
| Device | Cyclone V GX |
| Quartus version | QuartusII v15.0 |
| Modelsim version | ModelSim ALTERA STARTER EDITION 10.3d |
| Datarate | 1Gbps |
| Data pattern | Fixed and incremental |
| Number of channels | 1 |
| IP used | Custom PHY IP, Transceiver Reconfiguration Controller |