| Customer | NOKIA |
| --- | --- |
| Product PN | ALC3H-8DG62058BAAB |
| Product Description | NUBCM |
| Product SN | RT203703833 |
| Repair ID | RMA2021-0031 /RT4510557 |

Date of issue:  27.SEPTEMBRIE.2021

Product details:

Component details:

| Material PN | ALC3-1AB425040001_REP |
| --- | --- |
| Material Description | ICFPGA5CSXFC6C6U2317MCyclone5BGA/672333-I/O110-40C/100C |
| Material SN or Lot Nr | N/A |
| Date Code or Part Mark | N/A |
| Supplier Name | ALTERA |
| Supplier PN/MPN | 5CSXFC6C6U23I7N |

- Problem description: FPGA U1 (ALC3-1AB425040001) load failed

- Detected by: Customer field failure, failure confirmed by factory tests

- Quantity Affected: 1 PCS

- Failure Message: “Card failure-communication”

- Flex or Nokia analysis:MISSING PINS

This card failed at customer with no boot issue.

The nUBCM card failed at Prescreening test with “Card failure-communication” error. The failure is caused by FPGA U1 load failure and keeps repeating the load failure message continuously.

Replacing the component solved the problem and card passed factory tests.

Replacing the component solved the problem and card passed factory tests.

- Component Physical Condition:

7.Containment Plan

- Inspection mode: N/A

- Acceptable criteria: N\A

- Re-action: N\A

- Observation:    The pins on the component are missing.

- Inform component supplier about this problem - Supplier should investigate the cause of the failure