| Customer | NOKIA |
| --- | --- |
| Product PN | ALC3H-8DG60959ABAB_N |
| Product Description | PBA 100G SD FEC LINE DAUGHTER BOARD |
| Product SN | ST191750278 |
| Repair ID | GERTS089789 / ST193650249 |

Date of issue: 10.DECEMBRIE.2021

Product details:

Component details:

| Material PN | ALC3-1AB227640008_REP |
| --- | --- |
| Material Description | OBELIX 100 |
| Material SN or Lot Nr | N/A |
| Date Code or Part Mark | N/A |
| Supplier Name | ALTERA |
| Supplier PN/MPN | EP4S100G5H40I1N |

- Problem description: U9003 (1AB227640008) defective component

- Detected by: Customer field failure, failure confirmed by factory tests

- Quantity Affected: 1 PCS

- Failure Message: Signal Degrade Egress - ODU on PORT C1 at 25C

- Flex or Nokia analysis:

This unit failed in field at customer with “Signal Degrade Egress - ODU on PORT C1 at 25C” failure. Flex factory test confirmed this failure: Bit Errors from SSY at C1, pack works fine in Client Facility Loopback (with only client side active) but not in Client Far Loopback (client+obelix loopback). U9003 (1AB227640008) defective component.

Replacing the component solved the problem and card passed factory tests.

- Component Physical Condition:

7.Containment Plan

- Inspection mode: N/A

- Acceptable criteria: N\A

- Re-action: N\A

- Observation:    The pins on the component are missing.

- Inform component supplier about this problem - Supplier should investigate the cause of the failure